#### OMI-CS-25

INTERNATIONAL

## SINGLE PLANE HEART CATH LAB

## SYSTEM SPECIFICATIONS

# QTY.    CAT.NO.    DESCRIPTION

1	A-370	O-PIC 100 X-RAY GENERATOR, R/F, Cine & Angiographic

Applications

- Rating, 100 kW, Microprocessor Controlled

- Constant Potential Output Power

- Adult and Pediatric Mode Operation

- IsoWatt Operation for Optimum Contrast

- mA Stabilization

- Radiographic Ratings 1000mA@100kV, 630mA@125kV, 320mA@150kV

- X-ray Tube Overload Protection

- Automatic Brightness Control

- Automatic Exposure Control

- Automatic Calibration

- High Performance Variable Pulsed Fluoroscopy

- Integrated Anode Heat Unit Calculator

- High Dose Continuous Fluoroscopy

- 180 Hz Starter

- Fluoroscopic Dose Tracking

1	  B-200	ISOCENTRIC FLOOR MOUNTED L/C FRONTAL POSITIONER

- Motorized vertical SID	          10  inches

- Motorized rotation		              ± 90º

- Motorized angulation	               ± 90º

- Motorized Cranial/Caudal	             ± 50º

- Digital readout for L/C movements, table elevation, tilting degrees

1	  B-500	System Control - Single Plane

1	  C-300	ELEVATING CARDIO/ ANGIO TABLE

Tabletop:

- Four way floating carbon fiber top

- Transverse travel	              ± 6.5  inches

- Longitudinal travel		      48  inches

- Length		                104  inches

- Load Rating			    450 pounds

- Vertical travel	           33 - 41  inches

# QTY.       CAT.NO.   DESCRIPTION

C-300	ELEVATING CARDIO/ ANGIO TABLE, con’t.

Accessories:

1	   C-003	Transducer Pole & Mount

1	   C-004	2-Inch Thick Table Pad

1	   C-006	Two Arm Supports

1	   C-009	Table-Rail Mounted Radiation Shield

1	   D-405	SELECT HIGH CONTRAST/ LOW NOISE CARDIAC IMAGE

INTENSIFIER PACKAGE

- Triple Field  9”/ 6”/ 4.5” Tube & Power Supply

- Auto TV Rotation

- Auto Iris, I.I. Grid, Collision Device

2              D-505	18” Flat Panel Displays for Procedures Room

2				D-505	18” Flat Panel Displays for Control Room

1	   D-612	CEILING SUSPENSION for THREE (3) SIDE-BY-SIDE FLAT PANEL

DISPLAYS

- 13’4” Ceiling Rails

1	            E-615	PRINCESTAR PREMIUM HIGH RESOLUTION CARDIAC IMAGE

PACKAGE

- Advanced Large Format CCD Camera

- Integrated Auto Aperture/ Lens/ Mount

- Auto Calibration

- High Speed Xeon CPU - Internal RAID, 2 GB RAM

- 100 Base T and 1000 Base T Networking Points

- Ultra ATA System Drive

- RAID 10 Image Storage

- Storage, 48 Patient Studies (Studies, 10 Runs - 8 seconds - 30 fps)

- Auto Loop Replay

- Last Image Hold

- Real-Time 5 Level, 13 x 13 Edge Enhancement

- Auto DICOM Transfer to Network

- Image Processing 1024 x 1024 x 12 bit

- 9 Level Fluoro Noise Reduction Filters

- Pixel x Pixel Motion Correction

- LVA / QCA Measurement Software

- Programmable Fluoro and Cine Settings

- 15 or 30 Cine Frame Rates

- Zoom/ Pan/ Window/ Level Adjustments

- Annotation/ Pointer and Windows™ Print

- Static High Resolution 14 Image Display

- DICOM CD-RW with CV-10 Viewer

# QTY.      CAT.NO.   DESCRIPTION

1	   F-615	ROUND FIELD COLLIMATOR with LUNG FILTER

- Automatically collimates to selected field size

- Automatically tracks SID

1	   G-135	MGT HIGH SPEED DUAL FOCUS X-RAY TUBE with Fast Cooling Anode

- 1,500,000 heat unit anode

- 10 target, 270 horns

- Rated 18/ 112 kW

- 0.3 -1.0  f/s

1            G-500	QUIET STYLE HEAT EXCHANGER

- Reduces X-ray tube heat by 100,000 heat units per minute

1            H-800	Set of System Interconnecting Cables

1            H-806	Pair of Low Noise High Voltage Cables - 65 feet

1 	   I-101	Electronic Rack Cabinet

1		   I-110	CPU Electronics Enclosure

1	   Z-100	Complete Factory Pre-Staging of System

1	   Z-530	INTEGRATED FLUOROSCOPIC DOSE READOUT SYSTEM

- Displays accumulated dose per study

- Displays accumulated fluoroscopic exposure time per study

- Displays patient’s entrance dose rate

- End of study reset

Note:  Specifications subject to change without notification